# LED Interfacing Using 8051

## Aim:

To interface an LED with the 8051 microcontroller and control its operation.

## Apparatus Required:

1.Laptop with Keil uVision software
2.Proteus Design Suite

## Circuit Diagram Setup in Proteus:

- Open Proteus and create a new project.

- Add the following components from the library:
8051 Microcontroller (AT89C51)
LED
Resistor (330Ω)
Ground (GND) connection

- Connect the LED's anode to P1.0 of the microcontroller through a 330Ω resistor.

- Connect the cathode of the LED to GND.

- Save the design and proceed to programming in Keil.

## Algorithm:

- Configure P1.0 as an output port.

- Set P1.0 HIGH to turn ON the LED.

- Introduce a delay.

- Set P1.0 LOW to turn OFF the LED.

- Introduce a delay.

- Repeat the process continuously.

## Program:

Program (Keil - 8051 Assembly)

; led_blink.asm  - Blink LED on AT89C51 P1.0

; Assemble with Keil for AT89C51, produce HEX for Proteus simulation.

ORG 0000H       ; Reset vector

START:          MOV P1, #0FFH   ; Release Port1 (pull-ups) - make sure pins are high by default

CLR A

MAIN_LOOP:      SETB P1.0       ; Turn ON LED (assuming LED anode -> P1.0, cathode -> GND via resistor)

ACALL DELAY     ; Call delay

CLR P1.0        ; Turn OFF LED

ACALL DELAY     ; Call delay

SJMP MAIN_LOOP  ; Repeat forever

; -----------------------

; DELAY subroutine

; Nested loops using R7 (outer) and R6 (inner)

; Adjust values for longer/shorter delays

; -----------------------

DELAY:          MOV R7, #0FFH  ; Outer loop count (255)

DELAY_INNER:    MOV R6, #0FFH  ; Inner loop count (255)

DELAY_LOOP1:    DJNZ R6, DELAY_LOOP1

DJNZ R7, DELAY_INNER

RET

END

## Output:

KEIL OUTPUT:

PROTEUS OUTPUT:

## Result:

The LED interfacing with the 8051 microcontroller has been successfully implemented and simulated using Keil and Proteus.